Green Lane Community School

Asbestos Management Plan

September2024- 2025

Template Reviewed:

September 2024

VERSION:

6

DOCUMENT CONTROL

Record of Amendments

| DATE | AMENDMENT REFERENCE | VERSION | PAGES | AMENDMENT ACTIONED BY |
| --- | --- | --- | --- | --- |
| Nov 2013 | Written | 1 | 15 | D Watson |
| Sept 2015 | Template Reviewed - updated emergency procedures flowchart | 2 | 15 | D Watson |
| August 2017 | Template Reviewed – updated WBC logo | 3 | 18 | J Howard |
| March 2018 | Inserted Safe System of Work | 4 | 18 | D Watson |
| Nov 2021 | Template Review | 5 | 18 | D Watson |

| Contact Department | Email | Telephone ext. |
| --- | --- | --- |
| Community Safety and Resilience Team Health & Safety | Health&safety@warrington.gov.uk | 01925 442174 or 442681 |

Target audience

| All WBC staff | Y | Pupils | Y | Visitors | Y | Contractors | Y |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Contents Page Page Number

| 1. | Responsible Person and Premises Details | 4 |
| --- | --- | --- |
| 2. | Introduction | 4 |
| 3. | Responsibilities | 5 |
| 4. | Drawings / Plan (Location of Asbestos) | 6 |
| 5. | Asbestos Risk Register | 7 |
| 6. | Asbestos Containing Material Condition Inspection | 8 |
| 7. | Asbestos Action Plan | 9 |
| 8. | Communication | 10 |
| 9. | Safe System of Work | 13 |
| 10. | Risk Assessment | 14 |
| 11. | Emergency Procedures | 15 |
| 12. | Training Records | 18 |
| 13. | Annual Review | 18 |

Note

This Asbestos Management Plan must be adapted by Directorates and Schools to include relevant information relating to their premises and their activities.

1. 	Responsible Person and Premises Details

Responsible Person          Joanne Mullineux - Headteacher

Deputy and (Job Title)	Sarah Delaney – Deputy Headteacher

Premise 			Green Lane Community School

Address			Woolston Learning Village

Holes Lane

Woolston

Warrington

WA1 4LS

Date 				September 2024

List Relevant Surveys

2.	Introduction

Asbestos is a microscopic mineral fibre that is so small that it can’t be seen with the naked eye. Once disturbed, fibres can remain in the air for hours. If fibres are inhaled, they can cause serious health hazards which might not become apparent for years.

Click here to see what asbestos may looks like.

Asbestos should only be disturbed or maintained by specialist personnel working under strictly controlled conditions. Further information regarding maintenance or removal can be obtained from Building Services on Tel: 443506.

The Responsible Person has a ‘duty to manage’ the premises in respect of any asbestos it may contain. This requires knowing what asbestos containing materials (ACMs) may be present, their location and condition. The simplest way to do this is to have a survey undertaken by a qualified asbestos surveyor.  This can be sourced by Building Services, for further details contact Tel: 443506.

The contents of your survey can then be used to produce the management plan.  This should include regular checking of the condition of the ACMs and controlling any building or repair works to ensure this is not disturbed, except by specialists under controlled circumstances.

By completing the attached management plan will ensure that a consistent approach to Asbestos Management is being followed in all Council Buildings and the Warrington Family of Schools.

It is important that this document is read and completed in line with the Standard Operating Procedure SOP03 - The Management of the Risk from Asbestos in Civic Buildings and Schools.

3.  Responsibilities

This duty requires the Responsible Person (designated through the duty holder) to take the following key actions:

To ensure anyone who enters the premises (to include employees, pupils, contractors, service users and visitors, etc.) do not disturb ACMs and are safe from potential asbestos exposure.

Key point: Take note of any previous asbestos surveys undertaken, but don't rely that all asbestos materials within the building as being identified within them.  (There is the possibility that asbestos containing materials located on site may not have been detected during the survey.)

Duty Holders for the Premises

Joanne Mullineux - Headteacher

Responsible for the buildings and Health, Safety and Welfare matters.

For the provision of  selecting a competent contractor to undertake work in accordance with statutory provision, to include:- Control of Asbestos Regulation 2012, CDM 2015, Management of Health and Safety at Work Regulations 1999, HASAWA 1974, COSHH 2002, etc.  Also for ensuring that appropriate information is shared with their selected contractor prior to any intrusive works being undertaken.

Head Teacher

Responsible for monitoring and ensuring work is undertaken in accordance with the Control of Asbestos Regulations 2012.

Insert Name: Joanne Mullineux

Job Title: Headteacher

Responsible Person

Day to day management of ACM’s in their premises / building(s) to ensure Asbestos Management Plan and register is undertaken, maintained and communicated and that ACM’s identified are managed and controlled and any work undertaken on the premises is undertaken in accordance with the Control of Asbestos Regulations 2012.

Insert Name: Ian Wilcock (GLS Site Manager)

Gary Hall (FW Site manager)

Job Title: Site Manager

WBC Building Services

Via Equans (Foxwood) Outside Contractors (Green Lane)

.

Specialist Approved Licensed Asbestos Contractor:

Via Equans

Appointed Person

Insert responsibilities

Insert Name: …………………………………

Job Title: ……………………………………………

Notification of Licenced Work and Notifiable Non Licensed Work to:

The Health and Safety Executive

4.	Drawings / Plan (location of asbestos)

Insert Drawing Plan (marking location of identified or presumed asbestos) click here to see the HSE example of site plan

5.	Asbestos Risk Register

Click here to determine your material score and priority score

| Location Plan ID | Where on site | Product | How Much? | Surface Coating | Condition | How easy is access? | Asbestos Type | Comment | Material Score | Priority Score | Risk Rating H/M/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GLS – D Block | D block D47 ground floor corridor under staircase | Dust and debris | N/A | Debris | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – D Block | D block D47 ground floor corridor – | Blue floor tiles beneath modern fitted vinyl sheet flooring Floor tiles and bitumen | N/A | Floor tiles and bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – D Block | D block D04 Reception ground floor corridor – | Mastic sealant to ventilation ducting above ceiling tiles throughout level. | N/a | Mastic | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – D Block | D block ground floor corridor D46 | General dust and debris to underside of staircase | N/A | Debris | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – D Block | D block ground floor corridor D46 | Grey floor tiles beneath modern fitted vinyl sheet flooring | 1square m | Floor tiles and bitumen | Low damage | Moderate | Chrysotile |  | 1 | 1 | Very low risk |
| GLS – D Block | D block first floor corridor Redundant area | Bitumen felt visible to edge on 2 No. skylights | N/A | Felt | N/A | N/A | NAD |  | 0 | 0 | None |
| GL S – D Block | D block first floor corridor Redundant area | Exposed boarding clad to structural steel beam | N/A | Boarding | N/A | N/A | NAD | Material is modern and has been sampled for confirmation | 0 | 0 | None |
| GLS – D Block | D block ground floor corridor D44 – Cleaners Store | Bitumen wrap to redundant pipe to ceiling void above ceiling tiles | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – C Block | C block ground floor Headteacher C02 | Boarding remnants to wood batons above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – C Block | C block ground floor Headteacher C01 | Boarding remnants to wood batons above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – C Block | C block ground floor Classroom C26 | Boarding remnants to wood batons above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – C Block | C block ground floor Classroom C26 | Boarding remnants to wood batons above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |

| Location Plan ID | Where on site | Product | How Much? | Surface Coating | Condition | How easy is access? | Asbestos Type | Comment | Material Score | Priority Score | Risk Rating H/M/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GLS – C Block | C block ground floor Sensory Room C24 | Boarding remnants to wood batons above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – C Block | C block ground floor Sensory Room C24 | Cement mixed with straw forming roof above ceiling tiles extending above C18 to C26 | N/A | Cement | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – F Block | F block ground floor Hall F07 | Boarding to ceiling | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – F Block | F block ground floor Hall F07 | Boarding to ceiling | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – D Block | D block first floor Food Tech area D1.09 | Mastic sealant to ventilation ducting above ceiling tiles throughout level. | N/A | Mastic | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS Sports Hall | Sports Hall | Boarding with panels to expansion joints | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |

| Location Plan ID | Where on site | Product | How Much? | Surface Coating | Condition | How easy is access? | Asbestos Type | Comment | Material Score | Priority Score | Risk Rating H/M/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GLS External Roof 019 | External roof | Roofing felt | N/A | Roofing felt | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS External Roof 020 | External roof | Roofing felt | N/A | Roofing felt | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS External Roof 021 | External roof | Bitumen coating to sports hall roof | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS External Roof 022 | External roof | Old mastic sealant to original skylights of first floor roof | 35 square m | Mastic | Low damage | Difficult | Chrysotile |  | 1 | 1 | Very low risk |
| GLS External Roof 023 | External roof | Roofing felt to first floor roof | N/A | Roofing felt | N/A | N/A | NAD |  | 0 | 0 | None |
| GLS – F Block | F block ground floor Reprographics room F06 | Boarding to ceiling above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |

| Location Plan ID | Where on site | Product | How Much? | Surface Coating | Condition | How easy is access? | Asbestos Type | Comment | Material Score | Priority Score | Risk Rating H/M/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GLS E Block | E Block ground floor Room E15 | Mastic sealant to ventilation ducting above ceiling tiles throughout level. | N/A | Mastic | N/A | N/A | NAD |  | 0 | 0 | None |
| Sixth Form College | A Block – Ground Floor Staff Room A05 | Boarding to ceiling above ceiling tiles extending above rooms | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| Sixth Form College | A Block – Ground Floor Hygiene Room A23 | Boarding to ceiling above ceiling tiles extending above rooms | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| Sixth Form College | A Block – Ground Floor Classroom A09 | Boarding to ceiling above ceiling tiles extending above rooms A06 to A08 | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| Sixth Form College | A Block – Ground Floor Classroom A12 | Boarding to ceiling above ceiling tiles extending above rooms A11 to A12 | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |

| Location Plan ID | Where on site | Product | How Much? | Surface Coating | Condition | How easy is access? | Asbestos Type | Comment | Material Score | Priority Score | Risk Rating H/M/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sixth Form College | A Block – Ground Floor Quiet Room A16 | Boarding to ceiling above ceiling tiles extending above rooms A13 to A19 | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| Sixth Form College | A Block – Ground Floor Classroom A21 | Boarding to ceiling above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| Sixth Form College | A Block – Ground Floor Male W.C. A27 | Boarding to ceiling above ceiling tiles extending above rooms A26 to A29 | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |
| Sixth Form College | A Block – Ground Floor Classroom A30 | Boarding to ceiling above ceiling tiles | N/A | Boarding | N/A | N/A | NAD |  | 0 | 0 | None |

| Location Plan ID | Where on site | Product | How Much? | Surface Coating | Condition | How easy is access? | Asbestos Type | Comment | Material Score | Priority Score | Risk Rating H/M/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JM Block | G.02 Staff Room | Sink - Bitumen sink pad | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| JM Block | G.08 Classroom | Sink - Bitumen sink pad | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| JM Block | G.12 Classroom | Sink - Bitumen sink pad | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| JM Block | 1.04 Classroom | Sink - Bitumen sink pad | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| JM Block | 1.06 Kitchen | Sink - Bitumen sink pad | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| JM Block | 1.10 Classroom | Sink - Bitumen sink pad | N/A | Bitumen | N/A | N/A | NAD |  | 0 | 0 | None |
| JM Block | L.01 Loft void above first floor rooms | Ductwork - Mastic sealant to joints / sections | N/A | Mastic | N/A | N/A | NAD |  | 0 | 0 | None |

Completed or reviewed by: Ian Wilcock Date Sept 2024

# 6.	Asbestos Containing Material Condition Inspection

Good Practice Guidance:

- Take photographs of ACMs for each inspection and store for future reference

- Compare previous photographs with current condition to determine whether there has been any deterioration since the last inspection

- Where the condition is determined as either Medium or High, the emergency procedure needs to be instigated

Condition Key:

| Good: No visible damage; review at next inspection date | Low: A few scratches or surface marks but no signs of asbestos debris; review at next inspection date for further signs of deterioration |
| --- | --- |
| Medium: Significant breakage of materials or several small areas where material has been damaged; encapsulation works likely - follow the emergency procedure | High: Visible asbestos debris; environmental clean and encapsulation works likely – follow the emergency procedure |

| Location Plan ID | Where on Site | Asbestos Type | (delete as applicable) CONDITION | INSPECTION COMMENTS |
| --- | --- | --- | --- | --- |
| GLS – D block | D block – ground floor corridor D46 | Chrysotile | Low | Very low risk – Satisfactory condition – we will inspect periodically. |
| GLS – External roof | First floor skylight had old mastic seal | Chrysotile | Low | Very low risk – Satisfactory condition – we will inspect periodically. |

Inspected by: Ian Wilcock Date Sept 2023

7.	Asbestos Action Plan

Click here to see the HSE example Asbestos Management Plan)

| Location Plan ID | Where on site? | Product | Action to be taken? | By When? | By Whom? |
| --- | --- | --- | --- | --- | --- |
| GLS - D block ground floor corridor D46 | GLS - D block ground floor corridor D46 | Floor tiles and bitumen -Chysotile | Site manager to carry out annual checks. | Annually | Site manager |
| GLS – External roof | GLS – External roof | First floor skylight has old mastic sealant.- Chrysotile | Site manager to carry out annual checks. | Annually | Site manager |

Completed or reviewed by: Ian Wilcock Date Sept 2024

# 8.	Communication

# Asbestos location

# Insert how asbestos locations are to be identified on site (labels – colour coded stickers)

# Staff

All relevant staff will be provided with relevant information on:

- Types and location of ACMs (via this Asbestos Register and Plan)

- The visual means of identifying ACM’s (labels/colour coding)

- How to avoid risks from asbestos (e.g. not disturbing) any prohibited areas.

- How to report concerns about ACMs (e.g. to the Responsible Person)

- All new staff, through the Induction process, will be introduced to the Asbestos Management Plan.

- A safe system of working will be produced.

- Any concerns from staff will be reported to the Site manager or Headteacher

- A monthly check and an annual evaluation will be made around ACMs and these reports will be available for staff.

- The Headteacher and Site manager will receive regular training around asbestos.

- The safe system of working will identify and clarify what staff should or should not do around asbestos.

# Maintenance & Contractors

All Maintenance workers and contractors will be provided with relevant information on:

- The whereabouts and stability at ACMs via the school plan and the Asbestos Register and Management Plan.

- That monthly asbestos checks are carried out monthly and are dated and signed.

- Before any internal work is carried out in the school contractors are made aware of the Asbestos Management Plan and the location of ACMs.

- Where suspicion of ACMs is given we will request an appropriate survey before work is undertaken.

Other: Any relevant visitors, groups or clubs will be provided with relevant information.

Communications

# Contractor Review of Asbestos Register/Survey

This sheet should be signed by all those carrying out repair/maintenance work on the premises that will involve disturbing the fabric of the premises.

Persons signing this sheet are signing to say that they have seen the Asbestos Register and Management Plan and any relevant Surveys and checked that there is no known or presumed asbestos in the area in which they are working.

| Date | Company | Details of Work | Print Name | Signature |
| --- | --- | --- | --- | --- |

Where an asbestos containing material is suspected, no work must be carried out until the relevant procedures have been put in place to manage the risks of exposure.  This may require sampling to detect the presence of ACM’s or a Refurbishment Survey if multiple materials are to be disturbed, If ACM’s are detected removal may be required prior to any work being undertaken.

Communication

# Staff review of Asbestos Register Management Plan and Safe System of Work.

The following members of staff have received informed relating to asbestos risks within the premises (electronic register via google forms)

| Date | Name | Job Title | Signature |
| --- | --- | --- | --- |

Visitors, Groups or Clubs, Organisers review of Asbestos Register Management Plan and Safe System of Work.

The following Visitors or organisers of clubs or groups have received information relating to the management of asbestos of this premises.

| Date | Club or Group | Organiser name | Signature |
| --- | --- | --- | --- |

9.   	Safe System of Work

# Introduction

Green Lane Community Special School is a building that contains ACM’s (Asbestos Containing Materials). There is an Asbestos Register which contains information on where asbestos is presumed and also in areas where sampling has been undertaken and asbestos detected.  Whilst this gives a picture of where we have identified ACM’s in the premises, it is to be presumed that given the building age there may be other areas in the school where samples have not been taken which ‘may’ contain ACM’s and as such the following SSOW should be followed at all times.

- This is a building that has identified Asbestos in specific areas identified on the Asbestos register.

- An Asbestos Management plan including the register is held in reception and is available if staff/Visitors/contractors need to consult it at any time.

- An annual inspection of visible asbestos is undertaken by the Site Manager.

- The action that would most likely result in asbestos (known) fibres being released is disturbance of the building fabric and any fixtures and fittings. No disturbance of any known or presumed area must take place until contact has been made with the responsible person for day to day asbestos management, Site Manager.   This includes the disturbance of any unsampled areas where asbestos could still be in situ.

- In order to ensure the safety of staff and children and visitors, staff must regularly conduct their own visual inspection of the working environment and report immediately any areas that appear disturbed to the responsible person for day to day asbestos management, Site Manager.  This will allow a timely investigation to take place.

- Staff will be informed of the asbestos register and the importance of liaison with the responsible person via emails and staff briefings and a copy of this management plan will be available in reception.

- If staff have ANY concerns they must implement the emergency Asbestos isolation procedure. (Details attached.)

Staff must never put staples, pins or nails into walls, floors or ceilings before contacting the responsible person. If they wish to alter their rooms they must first seek guidance from the Site Manager or a member of staff who has been trained in Asbestos awareness.

10. 	Risk Assessment

| ### Directorate: Children’s Services | Location: Green Lane Community School |  | Date of Assessment: Sept 2024 |  | Review Date: Sept 2025 |
| --- | --- | --- | --- | --- | --- |
| #### Name of Assessor: Ian Wilcock |  | Position of Assessor: Site Manager |  | Signatures: Ian Wilcock |  |

#### Title and Ref. Number of Task – Asbestos Containing Materials (ACMs)

| #### Hazard | Persons at Risk | Potential Harm | Existing Control Measures | Risk Rating (H/M/L) | Further Action Required | Residual Risk (H/M/L) |
| --- | --- | --- | --- | --- | --- | --- |
| GLS - D block ground floor corridor D46 - Floor tiles and bitumen -Chrysotile | Pupil, Staff, visitors and workmen | Exposure to Asbestos. | Potential risk is very low. ACMs have been identified and will be monitored annually. Moderately difficult to access. | L | Ensure monitoring sheets are signed and dated. | L |
| GLS external roof. First floor skylight has old mastic sealant.- Chrysotile | Workmen – caretaker going on roof | Exposure to Asbestos. | Potential risk is very low. ACMs have been identified and will be monitored annually. They are on the roof and are difficult to access | L | Ensure monitoring sheets are signed and dated. | L |

11. 	Emergency Procedures (Asbestos Containing Materials)

If suspected ACM’s are discovered during work or become damaged by any other physical method, the risk assessment for work undertaken and the asbestos register for ACM’s should be readily available and all relevant personnel should be provided with information, instruction and training in the emergency procedures

- If you suspect ACM’s - STOP WORK IMMEDIATELY inform the Responsible Person who will commission Building Services or the designated licensed asbestos contractor to deal with the incident immediately and to make safe. The Community Safety and Resilience Team must be informed if ACM’s may have been disturbed along with details of the incident.

- Isolate the area i.e. shut doors and windows, turn off ventilation systems and fans if possible

- Post warning notices informing all staff to keep away

- DO NOT ALLOW any access into the area until it is made safe by an approved contractor

- Check register to establish whether ACM’s are present

- In circumstances where the register is not specific engage with Building Services / alternative competent licensed contractor immediately to sample the area

- If the analysis confirms that the material is none asbestos then work can continue

- If asbestos is discovered and confirmed then the contracted licensed asbestos removal company should be called to remove / repair the asbestos / if non licensed work is required then this should be determined before any work is undertaken and all work should be in accordance with current legislation before the room can be re-occupied (possibly with air clearance certification the asbestos contractor will provide guidance on what would be required).

- The Responsible Person must ensure that ACM’s are identified and managed before any planned maintenance is undertaken.

Damaged materials that could contain asbestos?

Stop work immediately!

Discovered materials that could contain asbestos?

Stop work immediately!

Emergency Procedures (Asbestos Containing Materials) flowchart taken from HSE EM1

Is there dust or debris on clothing?

Keep everyone else out of the area

Report the problem to the responsible person as soon as possible

A lot e.g. contaminated clothes, hair, and footwear.

A little, e.g. dust on sleeve, on shoes.

Stay put, avoid inhaling dust, put on RPE if possible, damp wipe outer clothing if possible, then remove outer clothing before moving away from source

Put up warning signs ‘possible asbestos contamination

‘Access Prohibited Do Not Enter’

Wipe down with damp rags.

Presume it contains the worst type of asbestos

Arrange a sample to be analysed

Seek help.  Helper to put on PPE and RPW before entering area

OR

Does it contain asbestos?

Damp wipe all contaminated items: clothing (as necessary), footwear and skin/hair until visually clean.

No

Yes

Apply appropriate controls using licensed contractor if required

No action required

Decide if need to wash hair and shower

Put contaminated clothes, towels etc. in a plastic bag and dispose of as asbestos waste

Update the Asbestos Management Plan

Dispose of rags as asbestos waste and keep a record of event

Report the event using the WBC accident / incident online reporting system

Decide if the work needs a licensed contractor

Emergency Procedures (Asbestos Containing Materials)

Follow the chart on page 16 and carry out a risk assessment to decide who must do the work, you may need a licensed contractor, minimise the spread of contamination to other areas to keep exposures as low as possible, clean up the contamination using an approved contractor in line with legislation.

No Council employee will be engaged to remove any ACM’s from any council building.  If licensed work is required then only licensed asbestos removal contractors may undertake this work and for non-licensed work this may be undertaken by approved contractors in accordance with the regulations, ensure appropriate notifications are made to the HSE.

DO NOT

Allow any work to continue on any materials which are suspected of containing asbestos, if suspected ACM’s have been damaged or disturbed during work (s) in progress, the materials should be:

- left in-situ,

- the work (s) suspended

- the area isolated pending further investigation

DO NOT

Attempt to take a sample, the actual act of sampling asbestos can expose the sampler to dangerous levels of asbestos fibres and the surrounding areas could also become contaminated.  Samples can only be taken by a UKAS accredited laboratory.

Should the material be found to contain asbestos, a scope of work must be agreed with the environmental analyst, asbestos removal contractor and contract administrator to repair or remove the material as soon as possible.

If licensed work is required the removal contractor will submit notification to the HSE where appropriate and the area should remain closed until the asbestos works are complete and the environmental analyst has issued the relevant certificate of reoccupation. It may be necessary to programme further asbestos removal works.

COPIES OF ALL REPORTS FROM SAMPLING TO REMOVAL MUST BE RETAINED WITH THE ASBESTOS MANAGEMENT PLAN IN THE ASBESTOS FILE

12. Training Records – Asbestos Management / Asbestos Awareness

| Name | Course Name | Course Provider | Date of Training | Signature |
| --- | --- | --- | --- | --- |
| Joanne Mullineux | Asbestos Management Training | SSS Learning | July 2024 |  |
| Sarah Delaney | Asbestos Management Training |  |  |  |
| Ian Wilcock | Asbestos Management Training | Adele Partridge (H&S in Care) | March 2023 |  |

13.	Annual Review

This asbestos management plan must be reviewed on an annual basis to ensure risk registers, risk assessments, training and the action plan are kept up to date.

Any changes to the management of asbestos in Civic Buildings and Community Schools must be communicated to all relevant parties identified within the Communication Section.